GAMIT GLOBK

Process for data collection and data processing

- Data Collection

In GAMIT, we navigate to the directory where we want to store the data and run the following command to download the data:

getgs -yr year -doy startday -end endday -sta station -rcv receiver -s 30 -i 30 -h 24 -f rinex -r 30s

This will download data for the specified year, start day, end day, station, and receiver, at 30-second intervals with 30-second overlap, using a 24-hour window. The data will be stored in RINEX format at 30-second sampling rate.

- Data Processing

- Process the data using GAMIT to generate the required files for GLOBK. Run the following commands in GAMIT:

editcsh

- This will open the editcsh script file. In this file, add the following lines:

gps2met

do_gps_grid

do_apriori

do_block

do_site

do_geoid

do_tropo

do_lave

do_ionex

do_gamit

- Now, run the following command to execute the script:

./editcsh

This will process the data and generate the required files for GLOBK.

- Use GLOBK to calculate the heights. Run the following command in the terminal:

height -f file

Here, "file" is the file containing the station coordinates and phase residuals, which should be generated during the GAMIT processing. This command will calculate the heights and output the results to the terminal.

Following the above steps the height data for various points over our desired point are obtained.

Further processing is as followed:

- The elevation of surfaces data is collected of various time points in a range of years.

- The collected data is converted to an excel format.

- The mean height of points within a specified area are calculated.

- The rate of change of these mean values of elevation with time is measured.

- With the trends of the previous rate,  the rate of change  of upcoming time  can be estimated.

The rate of fall down (Subsidence rate) can predict the possibility of subsidence in upcoming years.

The possible precautions then can be taken accordingly.

To display Elevation data on a web page using HTML and Python:

We use Python to convert the generated excel file to HTML table format:

import pandas as pd

# Read Excel file into a pandas DataFrame

df = pd.read_excel('data.xlsx')

# Convert DataFrame to HTML table

html_table = df.to_html()

# Write HTML table to file

with open('table.html', 'w') as f:

f.write(html_table)

HTML code to display in web page format

<!DOCTYPE html>

<html>

<head>

<title>Elevation</title>

<style>

table {

border-collapse: collapse;

width: 100%;

}

th, td {

text-align: left;

padding: 8px;

border-bottom: 1px solid #ddd;

}

tr:hover {

background-color: #f5f5f5;

}

</style>

</head>

<body>

<h1>My Excel to Website</h1>

<table>

<thead>

<tr>

<th>point</th>

<th>latitude</th>

<th>longitude</th>

<th>Height</th>

<th>Mean</th>

</tr>

</thead>

<tbody>

<?php

$file = fopen("mydata.csv","r");

while(! feof($file))

{

echo "<tr>";

$data = fgetcsv($file);

foreach ($data as $value) {

echo "<td>".$value."</td>";

}

echo "</tr>";

}

fclose($file);

?>

</tbody>

</table>

</body>

</html>

Following JavaScript is used to add search element to website.

// JavaScript

function search() {

const query = document.getElementById("searchInput").value.toLowerCase();

const results = document.getElementById("searchResults");

// Perform search logic here

// ...

// Display search results

results.innerHTML = "Search results: " + numResults;

}